RECREATIONAL VEHICLE (RV)

Combination Carbon Monoxide and Gas Alarm

USER’S   MANUAL

##### Models 70-742   70-742-MS   70-742-R   70-742-R-MS

PATENTS PENDING       ALL RIGHTS RESERVED 	70RV052004-2

ATTENTION: This user’s manual contains important Carbon Monoxide (CO) and gas alarm installation, operation, troubleshooting and warranty information. Read, follow, and keep this manual for future reference.

NOTE:  If you install or purchase this alarm for another person, give this manual to that person.

IMPORTANT: This unit must be replaced within five years of  its production date.  Record the manufacture date in the section, Owner’s Replacement  Record, in this manual for future use.  The replacement date indicates the date beyond which the device may no longer detect carbon monoxide, propane or natural gas accurately.  The device should be replaced.

Dual Sensor Technology

The SAFE-T-ALERT™ 70 Series combination CO / Gas Alarm is an alarm that combines into a single compact system, a powerful alarm that detects both Carbon Monoxide (CO) and explosive gases Propane (LPG) and Methane (Natural Gas). The 70 Series uses the latest microprocessor technology combined with two electronic self-cleaning sensors that operate independently of each other.  The combined unit can detect both CO and explosive gases simultaneously..

This detector is UL® listed as a CO Propane and Methane (Natural Gas) detector for RV use. Other explosive gases detected, but not tested by UL®, include Acetone, Alcohol, Butane, and Gasoline all of which you may have in your RV.

To prevent false alarms from brief “puffs” of gas the detector has a recheck cycle before alarming. If high levels of gas remain during the recheck cycle, the detector will alarm.

The electronic CO sensor in your safe-t-alerttm CO alarm is very sensitive to CO gas. It will not react to most other gas.

WHY EVERY RV NEEDS A 70 Series CO and Gas Alarm

Everyone is at risk for carbon monoxide poisoning!  Particularly sensitive are children, pregnant women, the elderly and people with lung or heart disease or anemia! Carbon monoxide (CO) is an odorless, colorless gas that prevents the blood from carrying oxygen to vital organs.  CO is 200 times more likely to replace oxygen in the blood. The Consumer Product Safety Commission (CPSC) recommends using at least one CO alarm located outside of sleeping areas. For the best security, locate a CO alarm in each sleeping area.  Safe-T-Alert CO alarms are available for secondary installations.

Propane gas can cause explosions!  Using propane requires taking safety precautions to avoid injury.  These gases is usually identified by an unpleasant odor. Certain conditions may make some people unable to detect this unpleasant odor.  These conditions include advanced age, colds, allergies, and the  use of tobacco, alcohol or drugs that may diminish the sense of smell.

# ABOUT THIS ALARM

Because CO is a colorless, odorless, tasteless and highly poisonous gas; it can endanger lives even at low levels of concentration.  The CO Alarm will alert you to potentially dangerous situations.  The following symptoms may be related to  CO POISONING.  Discuss these symptoms with ALL household members and RV guests:

- Mild Exposure: Headaches, running nose, sore or watery eyes, often described as “flu-like” symptoms.

- Medium Exposure: Dizziness, drowsiness, vomiting.

- Extreme Exposure: Unconsciousness, brain damage and death.

NOTE: Reported cases of gas poisoning indicate that while

victims are aware they are not well, they are disoriented.  They are unable to save themselves by exiting the RV or calling for

assistance.  Small children and pets may be affected first.

Your  safe-t-alerttm 70-742 alarm helps protect your household members and guests from CO produced while using your RV. CO gas is produced when any type of fuel is incompletely burned. Potential sources of CO  in and around your RV can include gas or diesel engine exhaust, portable space heaters, gas stoves and ovens, furnaces, defective engine exhaust systems, portable grills, other nearby RVs, portable generators, generator exhaust, and other propane-powered appliances. All are sources of CO.

The following are also sources of CO that may affect your RV:

- Extended operation of unvented fuel burning appliances can build up high CO levels.

- An idling vehicle in an open or closed garage.

- Temperature inversions can trap exhaust near the ground.

- CO build up can be caused by reverse/negative venting of fuel burning appliances including; 1) Clogged, loose or faulty stacks or chimneys of (clothes dryers, furnaces and water heaters, etc.), 2) wind direction and/or velocity, 3) simultaneous operation of multiple fuel burning appliances, and/or exhaust fans.

WARNING:

LIMITATIONS OF  CO and GAS ALARMS

THIS ALARM WILL NOT WORK WITHOUT POWER. Some reasons for no alarm power; are a blown or missing fuse, broken wire, a faulty wire connection or circuit breaker, a discharged battery, cut lead wires,   or improper supply (+) or ground (-) connections.

THIS ALARM WILL ONLY INDICATE THE PRESENCE OF GAS AT THE SENSOR. CO or explosive propane/natural gas gases may be present in other areas. MTI recommends installing CO alarms in all sleeping areas.

THIS ALARM IS INTENDED FOR USE IN  RV's.  It is intended for use in inside locations of a RV.  It is not designed to measure compliance with commercial and industrial standards.

THIS ALARM MAY NOT BE HEARD.  The alarm’s loudness is designed to meet or exceed regulatory standards; however, the alarm may not be heard if alarms are located in remote locations or behind closed doors.  Persons who are hard-of-hearing, have consumed alcoholic beverages, taken prescription, non-prescription or illegal drugs, may not hear the alarm.

THIS ---- ALARM IS DESIGNED TO DETECT CO FROM ANY SOURCE OF COMBUSTION AND PROPANE AND METHANE GASES. THE ALARM IS NOT DESIGNED TO DETECT SMOKE, OR FIRE.

THIS ---- ALARM  MAY NOT ALARM AT LOW CO LEVELS It is not designed to measure compliance with the Occupational Safety Health

Administration (OSHA) commercial or industrial standards. Individuals with medical problems may consider using warning devices, which provide audible and visual signals for CO concentrations under 30 ppm.

###### HOW TO PROTECT YOUR FAMILY

CAUTION- The safe-t-ALERTTM 70 series combination CO and Gas Alarm is designed to protect individuals from the acute effects of carbon monoxide exposure and gas leaks. It will not fully safeguard individuals with specific medical conditions. If in doubt, consult a medical practitioner.

To protect your family, you should:

- Install THE ALARM PROPERLY.  Carefully read and follow ALL the instructions in this manual.  Test your unit every week. Alarms that do not work will not alert you to hazardous levels of CO or explosive gas. See the section, Test Procedure, in this manual for further information.

- MAKE REGULAR VISUAL INSPECTIONS. Check all fuel burning equipment including gas water-heaters, kitchen gas stoves, space heaters, gas dryers and all pilot lights. Check the color of the pilot flame.  The color should be blue.

- MAKE REGULAR VISUAL INSPECTIONS of the engine and generator exhaust systems. Cracked exhaust systems can allow CO to enter the living area.

- PROFESSIONALLY MAINTAIN YOUR ENGINE AND GENERATOR. Although gas engines and generators produce CO, a poorly tuned engine and generator will produce greater amounts CO.

WHERE TO INSTALL

- Install the 70 series in the kitchen area near sources of a potential gas leaks. Some potential sources are a furnace, refrigerator, stove or oven. If potential sources of a gas leak are in separate areas, MTI recommends installing a gas alarm in each area.

- The Consumer Product Safety Commission recommends installing a CO alarm outside the sleeping area. . The 70 series combination CO and gas alarm complies with that recommendation. For added protection, MTI suggests installing a stand-alone CO alarm in the bedroom area.

- INSTALL ALARM no more than 20 inches off the floor.

------------

WHERE NOT TO INSTALL 70 SERIES ALARMS

- DO NOT INSTALL        --- behind furniture, drapes, in closets or areas that will block air flow to the alarm.

- DO NOT INSTALL within 12 inches (30cm) of opening windows, exterior doors, heating or return vents, or other drafty areas..

- =======

- -----------

- DO NOT INSTALL --- on an outside wall.

- DO NOT INSTALL --- on a wall switch controlled by a power line, ground fault circuit or to a circuit breaker.

- NOTE: Older RVs may have little or no insulation and therefore are draftier. Carefully consider mounting locations.

The following diagrams provide additional alarm installation information:

INSTALLATION INSTRUCTIONS

- Models 
70-742    70-742-MS   70-742-R   70-742-R-MS

- WARNING

FAILURE TO FOLLOW THESE INSTRUCTIONS MAY RESULT IN A MALFUNCTION OR FAILURE OF THE ALARM AND MAY VOID THE WARRANTY.

SHOCK HAZARD: Turn off power before installing.

Power Supply 12 vDC.

Recommended Wire Size 14 GA. TO 18GA. 

Connect the alarm to a properly fused circuit, maximum over protection device rating 15 amps. 

CONNECTING TO A MASTER STORAGE SWITCH

It is acceptable to connect the 70-741 and 70-742 propane alarms to master storage shut off switch. The alarm will be off along with all other 12 volt equipment when the storage switch is turned to the off position. Do not use the RV when the storage switch is in use.

All connections must be in accordance with the National Electrical Code in the Unites Stated and the Canadian Electric Code in Canada .

Connect to wiring or a circuit that CANNOT be turned off by a switch or ground fault protector.   Only use UL® or recognized connectors.

installation instructions

Follow these instructions carefully. Failure to follow these instructions can damage the unit and void the warranty.

Important: The 70 series case requires 1 3/8” (35mm) clearance.

Model 70-742 - Select a wall location no more than 20” off the floor.

- Cut a 4” (102mm) W x 3” (76mm) H hole.

- Connect Supply (+) to the Red wire lead.                     
Ground to Black (-) to the Black wire lead.

- Reconnect the power supply.

NOTE: All connections must be in accordance with the National Electrical Code in the Unites Stated and the Canadian Electric Code in Canada .

Model 70-742R  With Normally Closed single pole double throw relay. This unit supplies a 12 volt DC with 3 amp (max) output. The relay is energized whenever the alarm is operational. It may be used to shut down a generator or to open an (optional) gas solenoid valve. The relay may also be used to interface to alarm systems, using either the normally closed or normally open contact, depending on the requirements of the alarm system. In the event of an alarm the relay will shut off.  It will automatically energize after the unsafe levels of CO or gas clears. 
 Select a wall location no more than 20” off the floor.

- Cut a 4” (102mm) W x 3” (76mm) H hole.

- Connect the Red wire to the 12 volt supply (+).
Connect the Black wire to the 12 volt Ground (-)

3. Relay Output - White12 vDC Supply (+)
                           - Black/White Stripe Ground (-)
Note: Some devices do not require correct polarity and can be connected to either terminal or wire.

NOTE: All connections must be in accordance with the National Electrical Code in the Unites Stated and the Canadian Electric Code in Canada .

The 70-742-MS multi-station interconnect, (Maximum 10 units and 2000 feet of wire). which allows multiple 70-742-MS alarms to be connected together.  When one of the alarms detects unsafe levels of CO or   gas it will activate.  It will also activate the audible alarm of all the interconnected units.  Only the alarm that detected CO or   gas will have its Red LED light lit up so that you can determine the cause of the alarm.
 Select a wall location no more than 20” off the floor.

- Cut a 4” (102mm) W x 3” (76mm) H hole.

- Connect the Red wire to the 12 volt supply (+).
Connect the Black wire to the 12 volt Ground (-)

- Interconnect Wiring - Series (Daisy Chain) connect Blue to Blue and Yellow to Yellow.

NOTE: All connections must be in accordance with the National Electrical Code in the Unites Stated and the Canadian Electric Code in Canada .

.

Model 70-742-R-MS With Normally Closed single pole double throw relay and Multi Station Interconnect. This unit supplies a 12 volt DC  3 amp (max) output. The relay is energized whenever the alarm is operational. The 70-742-R-MS can also be interconnected with up to nine 70-742-MS units. It may be used to shut down a generator or to open an (optional) gas solenoid valve. The relay may also be used to interface to alarm systems, using either the normally closed or normally open contact, depending on the requirements of the alarm system. In the event of an alarm the relay will shut off.  It will automatically energize after the unsafe levels of CO or gas clears.

Select a wall location no more than 20” off the floor.

- Cut a 4” (102mm) W x 3” (76mm) H hole.

- Connect the Red wire to the 12 volt supply (+).
 Connect the Black wire to the 12 volt Ground (-)

- 3. Relay Output - White12 vDC Supply (+)
                      - Black/White Stripe Ground (-)
Note: Some devices do not require correct polarity and can be connected and can be connected to either terminal or wire.

- Interconnect Wiring – Series (Daisy Chain) Connect Blue to Blue and Yellow to Yellow.

NOTE: All connections must be in accordance with the National Electrical Code in the Unites Stated and the Canadian Electric Code in Canada .

TEST PROCEDURE

WARNING

TO REDUCE THE RISK OF CARBON MONOXIDE POISONING OR PROPANE GAS EXPLOSION, TEST THIS ALARM’S OPERATION AFTER THE RV HAS BEEN IN STORAGE, BEFORE EACH TRIP AND AT LEAST ONCE PER WEEK DURING USE.

- WARNING

- DO NOT USE A CIGARETTE LIGHTER TO TEST THE CO / Propane GAS ALARM. Butane gas may damage the sensors.

DO NOT attempt to test the alarm by any other means than by using the TEST/RESET button.  DO NOT attempt to produce CO - to test the alarm.  The TEST/RESET button tests all functions of the alarm and is the ONLY safe way to be sure the alarm is working properly.

The alarm may be tested at any time -The TEST/Mute switch is located on the front of the alarm Press and hold the test button for 1 second.  The alarm is working properly if the GREEN indicator light changes color to RED and the horn beeps 4 times. The Gas LED should also blink Red.

IMPORTANT -  If this alarm does not test properly return it immediately for repair or replacement.

OPERATION

--- When the unit is first powered up, the CO sensor requires a ten (10) minute initial warm-up period to clean the sensor element and achieve stabilization. The GREEN LED indicator will flash on and off during the 10 minute warm-up period. This unit cannot go into a CO alarm   during the warm-up period.  To test your unit during the warm-up period, press the test button. See Test Procedure in this manual.  After the warm-up period, the GREEN power ON indicator should glow continuously If the ON indicator light does not light, see the section, Trouble-Shooting Guide, in this manual for further information.  Do not attempt to fix it yourself.

Gas Alarm: When you power the alarm, it has a warm-up period of approximately 1 minute. This unit cannot go into a gas alarm during the warm-up period.  After 1 minute the alarm can detect explosive gas and will energize the relay on models 70-742-R and 70-742-R-MS.

Simultaneous CO and Gas Alarms— Because the risk of a propane gas explosion is generally a more serious danger, your alarm unit gives the gas alarm a higher priority during simultaneous alarm condition.

If your unit generates alarms for both Gas and CO at the same time, the gas LED --- will flash red and the beeper will sound.  The CO LED will be a solid Red until the CO is ventilated out of the RV, at which time the LED will return to the Green operational/safe color.

Brownout Protection - The unit can tolerate short power interruptions and brownouts where the circuit voltage drops as low as 1 VDC. If the brownout lasts too long, the unit will reset and operate as described above.

### LOW POWER OPERATION

This alarm will operate normally down to7 vDC.  Do not operate this alarm below 7 vDC.

VISUAL AND AUDIBLE ALARM SIGNALS

This safe-t-alerttm CO / Propane Gas Alarm is designed to be easy-to-operate. The alarm has two indicator lights that display a specific color for each monitored condition. There also is a matching sound pattern for alarm conditions.

CO ALARM

The Red CO LED will flash and  the alarm will sound  4  “beeps” then silent for 5 Seconds.  These signals indicates that the CO level is over 70 ppm. IMMEDIATE ACTION IS REQUIRED. See procedures to take during an alarm. This cycle will continue until the TEST/Mute button on the front of alarm is pressed. Ventilate the RV. The RED light will stay ON until the CO has cleared, or the alarm will reactivate in approximately 6 minutes if the CO is still present. DO NOT RE-ENTER THE RV. This alarm will return to normal operation after the RV s properly ventilated.

PROPANE GAS ALARM

The Red LED will Flash and the alarm will sound a steady tone whenever a dangerous level of propane or methane gas is detected. IMMEDIATE ACTION IS REQUIRED. See Procedures Take During A Gas Alarm The detector will continue to alarm until the Test/Mute switch on the front of the alarm is pressed. Ventilate the RV. The RED Gas LED will continue to flash until the gas has cleared, or the gas alarm will reactivate in approximately 5 minutes if the gas is still present. DO NOT RE-ENTER THE RV. This alarm will return to normal operation after the RV s properly ventilated.

MALFUNCTION/SERVICE SIGNAL If any malfunction is detected, the Gas LED will remain off and the Operational/CO LED will alternate Red/Green and the alarm will sound once every 15 seconds. Press the Test/Mute button. If the Test/Mute button does not clear the signals, check the battery voltage. If the battery voltage is not low and the unit will not return to normal operation, immediately remove the alarm and return for service or warranty replacement. See the warranty section in this manual.

| Operation Normal CO Alarm PROPANE ALARM aLARM Malfunction | AUDIBLE SIGNAL VISUAL SIGNAL none sTEADY Green 4 “Beeps” STEADy Red 5 Seconds Off “ CONSTANT FLASHING RED “ Beep” Every aLTERNATING 30 Seconds rED/gREEN |
| --- | --- |

MEMORY FEATURE – This alarm has a Peak Level Memory feature that remembers the approximate amount of CO that activated it. The memory feature does not record brief exposure to CO that would not activate the alarm.  This alarm will indicate one of four levels with chirps and blinks with the CO LED:

To activate alarm level memory, press the TEST/RESET button for less than 1 second.

- 1Chirp and 1 Green Blink = CO memory is clear

- 2 Chirps and 2 Red = below 100 ppm

- 3 Chirps and 3 Red Blinks = below 200 ppm

- 4 Chirps and 4 Red Blinks = above 200 ppm

----------

TROUBLE-SHOOTING GUIDE

Use this chart to trouble-shoot problems with this 70 series Alarm.

PROBLEM			CAUSE/SOLUTION

Green Operational LED Off	1.Wire Connection
	2. Reversed Wiring
	3. Main Power Off    		4. Missing or Blown Fuse
	5. Contact Customer Service

No Sound When Testing 	Contact Customer Service
No Red LED When Testing 	Contact Customer Service

Locked In Alarm		Contact Customer Service

MALFUNCTION ALARM LED	1. Check Battery Voltage

Red/Green - Beep Every 15 Seconds

## Battery Voltage is Good and it	2. Contact Customer Service

## Will Not Reset To Normal operation

Customer Service 800-383-0269    www.safetalert.com

HOW TO TAKE CARE OF YOUR ALARM

This CO / Propane Gas Alarm is designed to be as maintenance free as possible.  To keep your alarm in good working order, you must:

- TEST THE ALARM WEEKLY.  See the section, Test Procedure, in this manual for further information.

- VACUUM THE DUST OFF THE ALARM COVER. At least once a year (more frequently in dusty locations), use the soft brush attachment of your vacuum to clean the alarm cover.

- CLEAN THE ALARM COVER WHEN DIRTY.  Wash the alarm cover by hand. Use a cloth dampened in clean water.  Dry with a soft cloth.

- DO NOT SPRAY CLEANING AGENTS OR WAXES DIRECTLY ONTO THE FRONT PANEL.  This action may damage the sensor, cause an alarm or cause an alarm  malfunction.

- OBSERVE THE COLOR OF THE INDICATOR LIGHT.  At frequent intervals and during your weekly test, check the indicator light on the on the front panel of the alarm.

- See the section, Operation, in this manual for further information.

PROCEDURES TO TAKE DURING AN ALARM

Follow the alarm procedures for your country.  The emergency number depends on your travel location.

PROCEDURES TO TAKE

DURING A CO ALARM IN THE USA

- WARNING: Actuation of this device indicates the presence of carbon monoxide (CO) or propane gases, which can KILL YOU. If signal sounds (4 beeps and flashing or solid red light): 1) Operate the Test/Mute button; 2) Call your emergency local service (Phone Number _____________ ) (fire department or 911); 3) Immediately move to fresh air - outdoors or by an open door/window. Check that all persons are accounted for. Do not reenter the premises or move away from the open door / window until the emergency responders have arrived, the premises have been aired out and your alarm remains in its normal operation. 4) After following steps 1-3, if your alarm reactivates within a 24-hour period, repeat steps 1-3 and call a qualified appliance technician. (Phone Number _______________) to investigate for sources of CO from fuel burning equipment and appliances, and inspect for proper operation of this equipment. If problems are identified during this inspection have the equipment serviced immediately. Note any combustion equipment not inspected by the technician and consult the manufacturers’ instructions, or contact the manufacturer directly, for more information about CO safety and this equipment. Make sure that the motor vehicles are not, and have not been, operating in an attached garage or adjacent to the residence.

PROCEDURES TO TAKE

DURING A CO ALARM IN CANADA

WARNING: Actuation of this device indicates the presence of carbon monoxide (CO) and propane gases which can KILL YOU. If signal sounds (4 beeps and flashing or solid red light: 1) Operate the Test/Mute. Immediately move to fresh air - outdoors or by an open door/window. Check that all persons are accounted for. Do not reenter the premises or move away from the open door/window until the emergency responders have arrived, the premises have been aired out, and your alarm remains in its normal condition; 2) call your emergency local service (telephone number _________)

( Fire department or 911).

PROCEDURES TO TAKE DURING GAS ALARM

1. Turn off all Gas appliances (Stove, Heater, Furnace,
Refrigerator, etc.), extinguish all flames and smoking material.

2. Press the Test/Mute switch. DO NOT DISCONNECT  
    POWER

3. Evacuate the RV. Make sure everyone is accounted for.    
    Leave the door and windows open.

4. Turn off the propane tank valve.

5. Determine and repair the source of the leak. Get professional 
    help if necessary.

CAUTION- DO NOT RE-ENTER UNTIL THE PROBLEM IS CORRECTED.

SPECIFICATIONS

MODELS	70-742, 70-742-MS, 70-742-R, 70-742-R-MS

POWER SUPPLY	12 vDC

NOMINAL CURRENT DRAW                     	108 mA  	
                                                                         	 
OPERATIONAL  TEMPERATURE	-40o F to +150o F     -40o C to +66o C

RELATIVE HUMIDITY	15% (+/- 5%)  to 95% (+/- 4%)

AUDIBLE OUTPUT	85 dB @ 10 feet

GAS ALARM TRIGGER	< 25% of the LEL of Propane and Methane 
GAS LEVEL CONFIRMATION   	8 Second

CO ALARM TRIGGER		<15 Minutes @ 400 ppm

case DIMENSIONS	6.5”W x 3.5”H x 1.38”D
 	 
WARRANTY	1 Year Limited 

TESTING LABORATORY	Underwriters Laboratories
STANDARD	UL 1484, Third Edition Gas Detectors	         For Residential and RV Use	UL® 2034 Rev.  November 14, 2003
                                                                      ULc –CSA6,19-01

Owners replacement Record

Replace this safe-t-ALERTTM 70 Series CO /   Gas propane alarm within 5 years, of the production date on the back of the alarm.

Production Date ______________________

Replace Date   ______________________

Installer       ________________________

NOTE: The replacement date indicates the date beyond which the device may no longer detect accurately.

#### LIMITED PRODUCTS WARRANTY

MTI INDUSTRIES, INC. warrants to the original retail purchaser that its products will be free from defects of material or workmanship for a period of One (1) year from the date of retail purchase. If proven to have been defective in original materials or workmanship and returned, delivery costs prepaid, MTI INDUSTRIES, INC. will replace this product free of charge.

# LIMITS OF WARRANTY

Replacement is your exclusive remedy under this limited warranty or any other warranty (including any implied warranty of merchantability for a particular purpose).  Any and all implied warranties or merchantability or fitness for a particular purpose shall be limited to the warranty period from the original date of retail purchase.  MTI INDUSTRIES, INC., its dealers and distributors shall in no case be responsible or in any way liable for any incidental or consequential damages for any reason. Some states do not allow the limitation or exclusion of incidental or consequential damages, or allow limitations on how long an implied warranty lasts, so the above

limitations may not apply to you.  This warranty gives you specific rights, and you may also have other rights, which may vary, from state to state.

##### Product Not WARRANTED

NOTE- There are no user serviceable parts inside the case. Opening any safe-t-alerttm  product for any reason voids the warranty. This warranty does not cover damage or failure resulting from acts of God, abuse, misuse, neglect, or faulty installation.

warranty Return procedures

It is MTI’s experience that a CO / Propane Gas Alarm is sounding for a reason. Call, ask your dealer to call, or e-mail our Customer Service Department (as listed below) to trouble shoot the situation.

Customer Service Phone No. - 800-383-0269

Fax  No. 847-546-9007

E-mail Service@mtiindustries.com

Web Site: www.mtiindustries.com

If Customer Service determines that the unit is defective, a Replacement Authorization (RA) number will be issued.  No product will be accepted for service or replacement without first obtaining a RA number.

If authorized, return this product to:

MTI Industries, Inc. 
Warranty Dept. RA # (INSERT RA NUMBER HERE)
31632 N. Ellis Drive Unit 301 
Volo, IL 60073

NOTE:  Mark the RA number in the area shown on the outside of the box!

NOTE:

70-742 is the basic unit with no interconnection, no relay.

70-742-MS is the basic unit with interconnection no relay.

70-742R-MS is the Multi-station unit (interconnect) and relay.

70-742R is the basic unit with a relay no interconnects.

© MTI INDUSTRIES 2004