Derived from TrailManor Owners’ experiences

Compiled by: Ed Felker

Editor’s Note:

The following checklists were developed by collating and assembling the suggestions, ideas, and recommendations of numerous TM Owners from the TrailManor Owners Website: (http://www.trailmanorowners.com/yabbse/index.php?board=;action=logout;sesc=c1c9ae03b9fd2988eb2b1251741a2c1d).  Many threads were analyzed and the best of the suggestions were compiled.  In some cases specific individuals are cited.  In many, the contributors are not.  TM Owners contribute their suggestions and comments to the website with the sole purpose of helping other owners.  Compiling these checklists in no way transfers intellectual ownership from the original contributors to the editor.  These checklists represent a composite and ready resource of material on the hundreds of contributions to the TM Owners website.  It is intended to be a compendium, not representation of intellectual originality not that of the editor’s to claim.

The steps in these checklists do not represent the opinion, position, or endorsement of the TM factory.  They represent the opinion of the sources from which they came… owners of new and older Trailmanors.  The factory assumes no responsibility for the accuracy of the information contained herein.  They are intended as a guide only, and do not represent empirical data from the designers or manufacturers of the product.  The TM Company assumes no responsibility for their accuracy.

# Towing

### Towing Preparation

(  )  Mirrors: MOUNTED

(  )  Tow Bar & Trailer: INSPECT for loose bolts & worn parts

(  )  Tighten loose bolts/replace worn parts.  If bolts consistently loosen, use Locktite or a double nut to keep them tight.

(  )  Tire Pressure: CHECK & ADJUST (TM, TV)

(  )  Propane: OFF

(  )  Electrical accessories: OFF

(  )  Lights: OFF

(  )  Lug Nuts: TIGHT (TM, TV)

(  )  Wiring: CHECK, CONNECTED not touching ground, loose enough to make turns without disconnecting or damaging wires.

(  )  Running Lights, Brake Lights, Turn Signals. Hazard Lights: CHECKED, WORKING

Towing Hookup

(  )  Hook up on flat smooth surface.

(  )  Coupler & ball: CHECKED

(  )  Hookup weight distributing hitch. (Reese Trunnion Style W/D)

(  )  Pick reference points on TV front & rear wheel wells.  Measure & record distance to pavement.

Front  _____      Rear  _______

(  )  Chains to spring bars: ATTACHED

(  )  Coupler: LOWER onto ball and LATCH

(  )  Bottom knob of trunnion: SET into lower socket of ball mount & SLIDE knob into top slot of ball mount.

(  )  Snap up brackets: POSITIONED on “A” frame so chain is vertical.  TIGHTEN bolt securing brackets to frame.

(  )  Raise trailer tongue & TV rear with tongue jack.  Lower yoke of snap up bracket until it’s parallel with the ground, and slip the closest link over the hook (if there are less than 5 links between hook & U-bolt adjust ball mount angle rearward & repeat procedure).  With snap up bracket handle over the yoke, raise until yoke has passed “over center.”  Slide safety pin through the small hole to lock yoke in place.  Repeat other side.

(  )   Lower jack.  Re-measure front & rear wheel well distances.  Preferred that rear of TV settle same or more than front.

If Front settled more than Rear -- increase links between yoke hook & spring bar.

If Rear settled more than Front -- reduce links.

(  )  Check TM level.  Readjust ball mount angle or position.

(  )  Ensure bar & chain can move when turning corner.

(  )  Attach safety chains.  Cross cables & wrap around tow bar legs to keep from dragging.

(  )  Check security of hutch, coupler, & other equipment.

(  )  Attach electrical cable.

(  )  Lights function both vehicles.

(  )  Raise tongue jack & stow handle.

(  )  Check brakes and adjust brake controller if necessary.

(  )  Check  all items securely fastened.

(  )  LAST CHECK -- trailer jack, tongue support, and all four stabilizers are raised & locked.

(  )  Remove wheel chocks & stow.

(  )  Check side and rear view mirrors for good visibility.

Other considerations

-  Check maps and routes.

-  Drive with care and don’t forget there is a trailer behind you.

-  Each time you stop, check TM & TV; tow bar, base plate, cables, tires.  Make sure TM wheels are not hot to touch.  If they are, you have a brake or bearing problem.

-  Each day before you depart, check lights.

-  Make sure you have a second key for TM & TV.

-  Never allow yourself to be interrupted while hooking up.

-  Between trips, clean tow bar & cables.  Clean and lubricate all parts of the tow hitch as required.

-  USE & FOLLOW CHECKLISTS.

Pulling Out

(  )  Ensure levelers are stowed.

(  )  Throw out last of trash.

(  )  Take one last look at hitch & lights.

(  )  Take one last look around site.  Did you forget anything?  Is it clean?

(  )  Pull out of site slowly.

Daily Hitch maintenance when towing

(  )  Drop of oil at trunnion contact areas with ball mount.

(  )  Clean ball & coupler sockets and coat ball with grease

(  )  Check spring bar chains and U-bolts for wear.

(  )  Check that all bolts are properly tightened and hitch pin and clip are securely in place.

(  )  Check electrical hookups, all lights work, and safety chairs are connected properly.

W/D Hitch Lubrication

These procedures apply to the Reese Trunnion style W/D hitch.

(  )  Lube ball mount sockets.

(  )  Lube spring bar trunnions.  During hookup, place 1 drop oil on the top and a second drop on forward side of upper trunnion.  Place a third drop on the rear side of the lower trunnion before inserting trunnion into ball mount.  (Use a heavy oil such as Reese on the Ball.  Lube trunnions each towing day.

Lubricate lower trunnion with one drop at contact point between trunnion and lower socket.

(  )  Lube hitch ball with 1 or 2 drops of oil.

It is not necessary to unhook the spring bars; however, as there are two oil holes in the ball mount top plate for upper trunnion lubrication.  Excess oil, dirt, and grit should be wiped off whenever uncoupled.

SETUP (TrailManor & Campsite)

Disconnect Trailer

(  )  Trailer: CHOCKED

(  )  TV to TM Electrical Connection: DISCONNECTED

(  )  Emergency Breakaway Lanyard: RELEASED

(  )  Hitch: DISCONNECT

Open Front Shell

[A]

[B]

[A]

(  )  Four Corner Draw Latches [A] (white) two each side near the front and rear corners (both sides): RELEASE

(  )  Four Hold Down Latches [B] (black handles) two midway each side; 1 each side for front section, 1 each side for rear section: RELEASE (roofs will spring upward a few inches when you unlatch these pins)

(  )  Front shell (from front): PULL OPEN  (Firmly grip the bottom front edge of the front roof section at an off-center point; never grasp or pull from the center of the section.  Pull straight out gently, don’t lift; as you pull out, the roof section should  raise.  Don’t pull or exert pressure on the front window screen!)

(  )  Continue pulling: RAISE SECTION FULLY

[C]

(  )  Front bed: PULL OUT as far as it will go  (bed support braces will swing into position as you pull bed forward)

(  ) Front Corner Latches [C] (one on each side): SWING UP  Align pin with hole in square support bar and insert on each side.

(  )  FRONT SLIDEOUT MODELS ONLY: After lifting roof section & securing with the corner latches and square aluminum support tubes, grasp handle above gas bottle, & pull living room out toward you.

(  )  Window Rock Guard: RAISE

Open Rear Shell

Verify rear corner draw latches and side hold down latches were released previously in step 1 above.

(  )  Rear shell (from rear): PULL OUT same as you did for front using off center grip and outward pull like in step 3 above.

(  )  Rear Bed: PULL OUT as far as it will go (Bed support braces will swing into position) like in step 5 above.

(  ) Rear corner latches & stabilizer bars [G]: SWING UP.  Align and insert pin into a hole in matching bar just like was done for the front section in step 6 above.

(  )  Window Rock Guard: RAISED

Configure Door

(  )  Steps: LOWER

(  )  Upper Door: UNLATCH, lock to side of trailer

[G]

(  )  Lower Door: UNLATCH lower door clip

(  )  Door Jamb: PUSH OUTWARD into place. Insert pin sticking out of top of door into Spring Clip on door post.

(  )   Two door sections: ALIGN and LOCK TOGETHER using the rotating Door Tab (white rectangular plastic) onto the clip.

(  )  Left section of Door Jam: INSERT INTO SPRING

(  )  Insulated Door Jamb Flap: SECURED

(  )  Check that door closes

Level Trailer

Note: This leveling procedure applies to newer TM’s where the stabilizers can support the weight of the trailer.  Older TM’s should use the alternate procedure using leveling blocks. (See TM Tips Checklist)

(  )  Front-to-back: LEVEL first using tongue jack (wood block under tongue jack post
(  )  Place wood block under tongue jack post
(  )  Side to side: CHECK level  
(  )  Stabilizers: CRANK DOWN (start at rear, crank down low side rear corner stabilizer first until level. Place wood clock under stabilizer if necessary. 
(  )  Side-to-side: CHECK level
(  )  High side front stabilizer: CRANK DOWN until it bears weight

(  )  Side-to-side: RECHECK level 
(  )  Remaining Stabilizers: CRANK DOWN until they just take the trailers weight

(  )  Both axis: RECHECK level 
(  )  Wheels: CHOCK

Outside

(  )  Shore Power main CB: OFF

(  )  Shore AC Power Cable: CONNECT

(  )  Shore power AC CB: ON

(  )  Sewer line: CONNECT

(  )  Gray Water Tank: DRAIN if any water was left in.  Do not leave GRAY valve open to prevent sewer gas from getting into TM.  Only open valve when periodically draining Gray water tank.

(  )  City Water & pressure regulator: CONNECT & turn ON

(  )  LP Tanks: OPEN

(  )  Awning: LOWER and STAKE

(  )  Set up outside furniture, awning lights, carpet, etc.

Inside

(  )  FRONT Red Bed Pin: INSERTED into matching holes (each side of sliding bed tray)

CAUTION: Do not climb on beds until pins are inserted in locked position

(  )  Insulated Side Flaps: LOWER & SECURE using Velcro

(  )  Raise Wardrobe

(  )  Clearance: ENSURE ADEQUACY by pressing right ankle slightly against lowered bathroom wall

(  )  Wardrobe Handle: GRIP with right hand

(  )  Wardrobe: RAISE and TILT SLIGHTLY BACKWARD

(  )  Left hand: BETWEEN Top wardrobe and ceiling to prevent ceiling damage

(  )  Final Position: SLIDE to final position ON COUNTER  (prevent it from sliding on counter top)

(  )  Raise Bathroom

(  )  Hinged bathroom corner door: CAREFULLY RAISE and SLIDE TOWARD OUTER WALL

(  )  Two  door parts: LATCHED

(  )  Hinged folded bathroom walls: HOLD TOGETHER and RAISE TO VERTICAL

(  )  Hinged wall: SWING TOWARDS YOU to form a corner/wall and LATCH in TWO PLACES (to lower section of wall and to bathroom door header)

(  )  Rear red bed pin: INSERTED into hole in bathroom wall

(  )  Refrigerator Power: RESELECT

(  )  Refrigerator DC Fan: OFF

(  )  Hanging Cabinet: MOUNT

(  )  Insulated Flaps: LOWERED and SECURED

(  )  Heat &-or A/C: ON & SET as required

(  )  Water pump: ON

(  )  Water heater: ON

(  )  Roof Vents: SET

(  )  Toilet: CHARGED

(  )  Antenna: RAISE;    Booster: ON

(  )  TV & microwave: POSITION & CONNECT

# TEAR DOWN (Pre-trip):

# Breaking Camp.  Assumes TM is setup & extended.

# INTERIOR

# (  )  Outside Sewer Valve: CLOSED; then PULL inside toilet DUMP valve.  Clean toilet, & dump water one more time.

(  )  Loose items (coffee maker, TV, microwave, etc.): STOW

(  )  Refrigerator travel bars: POSITIONED

(  )  Cabinet loose items: CHECKED

(  )  Dismantle Bathroom

(  )  Shower curtain: STOW on rear bed

(  )  Rear Red Bed Pin: RETRACTED

(  )  Bathroom wall: UNLATCHED and SWUNG UPWARD

(  )  Folded bathroom walls: HOLD FIRMLY together and lay them in bathroom area

(  )  Door sections: UNHOOK and PULL the front wall/door inward a few inches

(  )  Door/Wall: LAY in bathroom area

# (  )  Store Wardrobe

(  )  Left Hand: PLACE BETWEEN wardrobe & ceiling

(  )  Wardrobe: TIP BACKWARD slightly with right hand, grasp base with right hand, and slide cabinet fully over the center isle

(  )  Front end: LOWER a bit and reach up with your right hand and GRIP HANDLE located on top of wardrobe

(  )  Lowered bathroom wall: PRESS RIGHT ANKLE AGAINST to ensure adequate clearance while lowering wardrobe

(  )  Wardrobe: LOWER onto its side on the floor

(  )  Dismantle Front

(  )  Insulated Flaps: RAISE and SERCURE to walls

(  )  Red Bed Pins: RETRACTED from holes

(  )  Front Curtains: PULLED to CENTER

(  )  Seat Cushions: LAY FLAT

# (  )  Bedding: TUCKED INSIDE bed frames

# (  )  Cabinet doors and drawers: CLOSED

# (  )  Hanging cabinet: DISMOUNT & stowed.

# (  )  Drapes: OPEN & TIED BACK

# (  )  Windows: CLOSED

# (  )  Interior lights: OFF

# (  )  Porch light: OFF

# (  )  Night lights: UNPLUGGED & stowed

# (  )  Water purifier: In container in sink?

# (  )  All gear below bed line

# (  )  Refrigerator Power Selection: DC

# (  )  Fridge DC fan: ON

# (  )  Water heater: OFF; tank drained

# (  )  Stove: OFF

# (  )  Heater and/or A/C: OFF

# (  )  Water pump: OFF

# (  )  Antenna: DOWN;     Booster: OFF

# (  )  Upper screen door: REMOVED

# (  )  Kitchen gear: STOWED

(  )  Distribute weight inside for optimal towing.  Keep weight distributed so that the bulk is carried by the axle and hitch ball.  Keep amount of items carried at the rear of the trailer to a minimum.  Heavy items at rear will induce sway while towing.

(  )  Double check for folding clearance

(  )  Steps: FOLDED

(  )  Doors: SEPARATED;  Lower in TRAVEL position, Upper latched OPEN?

EXTERIOR

(  )  LP Tanks: OFF

# (  )  Battery wiring: CHECK

# (  )  Battery condition: CHECK water in cells; voltmeter 13 - 14 volts

# (  )  Front rock guard: LOCKED

# (  )  Tongue & Hitch:  LUBED

# (  )  Safety Chain Latching pin: ATTACHED

# (  )  7 pin umbilical cord: CHECK & contacts CLEAN

# (  )  Trailer brake release cable: CHECK

# (  )  Draw bar handle: STOWED in the tow vehicle

# (  )  Black draw bar receiver hooks: TIGHT on frame

# (  )  Awning Lights: STOWED

# (  )  Outside items (chairs, grill, carpet, flag, etc.): STOWED

# (  )  Awning:  RETRACT;  handle & center span stored under bed

# (  )  Trailer brakes: CHECK

(  )  Wheel lugs: TIGHT

# (  )  Wheel bearings: LUBED

# (  )  Tires (plus spare): CHECKED; inflated to 50 psi?

# (  )  Stabilizer jacks: RETRACT UP & LUBED

# (  )  Stabilizer hand crank: STOWED

# (  )  Ensure sewer line connected and supported.

# (  )  Black water valve: OPEN; wait until black water drains

# (  )  GRAY water valve: OPEN (use gray water to rinse sewer line)

# (  )  Plumbing: LEAK FREE

(  )  External plumbing valves: CLOSED under trailer

(  )  Sewer line: CAPPED

(  )  Tank fill connection: CLOSED

# (  )  City water hose & Regulator: DISCONNECT & STOW

# (  )  Gray & black water valves: CLOSE

# (  )  Sewer Line: DISCONNECT, RINSE with garden hose, STOW

(  )  Rear tool box:  CLOSE and LATCH

# (  )  AC cord: DISCONNECT & STOW

# (  )  Cable TV & telephone line if used: DISCONNECT & STOW

# (  )  AC surge protector: DISCONNECT & STOW

# (  )  Hold down clamps: CHECK

# (  )  Roof vents: CLOSED

# (  )  TV antenna: DOWN;   Booster: OFF

(  )  Towing mirrors: INSTALLED

(  )  Steps: FOLDED

TEAR DOWN (Take down/Collapse Unit)

Note: Refer to the TM illustrations in the Setup Checklist

(  )  Store Door

(  )  Door: SWING WIDE OPEN and attach to the door catch found on the side of the TM body

(  )  Door Tab: ROTATE and separate door halves

(  )  Lower Door: GRIP by knobs and PULL to release the Door Jamb Pin from the spring clip

(  )  Lower Door: SWING INTO INDENTED travel position in doorway

(  )  Insulated Flap: RELEASE and FOLD onto door jamb

(  )  Door Jamb: PULL TO RELEASE the Door Jamb Pin

(  )  Left Door Jamb: SWING INWARD to travel position and use eye hook to latch it to the lower section of door

(  )  Lower Rear Shell

(  )  Rear Bed: SLIDE closed

(  )  Bed Stabilizing Bars: RELEASE & SWING FULLY DOWNWARD (both sides) so that the roof has clearance to lower

(  )  Roof: PUSH FORWARD (not down) into its lowered position

(  )  Roof Stirrup (Door Side): beginning with DOOR SIDE, place FOOT IN STIRRUP (side of trailer) and PUSH DOWNWARD with enough force to lock the Side Hold Down Latch Pin in the Black Roof Latch

(  )  Roof Stirrup (Street Side): REPEAT same as Door Side

(  )  Top Corner Draw Latches (White): ATTACH ROOF to Chassis by closing latches (one on each side near rear corner)

(  )  Sweep dirt & water off of rear shell roof.  Stand on hitch to reach front portion of shell

(  )  Wipe down seal at front of rear shell.  Helps keep from marking front ceiling.

(  )  Lower Front Shell

(  )  Front Bed: SLIDE INWARD to close

(  )  Bed Stabilizing Bars: RELEASE & SWING FULLY DOWNWARD (both sides) for roof clearance

(  )  Roof: PUSH REARWARD to lower

(  )  Roof Stirrup (Door Side): beginning with DOOR SIDE, place FOOT IN STIRRUP (side of trailer) and PUSH DOWNWARD to latch Side Hold Down Latch Pin in the Black Roof Latch

(  )  Roof Stirrup (Street Side): REPEAT same procedure as Door Side

(  )  Top Corner Draw Latches (White): ATTACH ROOF to Chassis by closing top latches near each side corner

(  )  Upper Door: CLOSE and LATCH in travel position

(  )  Hitch TM to TV: See “Towing and Hitch Checklist”

(  )  Hitch & Weight Dist Arms: CONNECTED

(  )  Car to Trailer Electrical connection: CONNECTED

(  )  Emergency Breakaway Lanyard: CONNECTED

(  )  Trailer Lights: CHECKED

(  )  Wheel Chocks: PULL & STOW.  Move TM forward off levelers.

(  )  Levelers: STOW

(  )  Take one last walk around.

# Equipping

Must Haves

(  )  Water Pressure Regulator

(  )  Water filters (whole house on inlet hookup for sediment, faucet filter in kitchen for drinking water)

(  )  Fresh Water Hose

(  )  In-Line Water Filter

(  )  Sewer Hose (short & long pieces -- 10’ & 20’ pieces)

(  )  Electrical Extension Cord (for inside trailer)

(  )  Electrical Adapters - A range of adapters to be able to plug your TM into whatever electrical service is available.  A 30 amp male to 15 amp female is a good start for a TM.

(  )  Garden Hose (for clean up & sewer hose rinsing)

(  )  Wheel Chock

(  )  Leveling Blocks - The orange Lynx Levelers work great, but any pieces of wood cut into squares will do just fine.  Don't depend on your stabilizer jacks to level your TM (older TM).  They are meant for stabilizing, not leveling.  New TMs specify leveling with scissors jack)

(  )  Level or Stick-On Levels (stick-on on all 4 corners)

(  )  Stabilizer blocks

(  )  Extra Fuses - Check your TM manual for the types and sizes of fuses and carry extras of each.  Toilet, in-line battery, and converter are a good start.

(  )  Extra Bulbs

(  )  Fire extinguisher (TM OEM)

(  )  First Aid Kit

(  )  Black Water Chemicals - Don't even think about using your toilet without adding waste chemicals first; trust me!  For the TM Thetford recirculating toilet use liquid Thetford Campa Chem Natural (6 bottles to the box @ WalMart).  Use 1 bottle per charge.  Charge toilet to slightly below charge line, be sure that toilet pump primes.  Also add 1 to 2 tablets of Potty Toddy tablets per charge.  A charge should last 4 days for 3 adults before requiring dumping.  If you are sensitive to the chemical smell, try Odorlos instead of the Campa Chem.

(  )  RV Toilet Paper - the degradable type used for RVs and portable toilets.  Using regular toilet paper will clog your black holding tank eventually.  Regular Scott’s is a good substitute.

(  )  Rags

(  )  Disposable Rubber gloves (never touch a sewer connection without these)

(  )  Non-Slip Cabinet Linings

(  )  Flashlight

(  )  Matches (and/or butane grill lighter)

(  )  Hitch Lubricant – Heavy oil & grease for daily lube of hitch components.

(  )  Metal stakes and nylon rope

(  )  Tools - Make sure you carry the basics, such as:

- Cheap rechargeable 3/8 drill 	- Needle-Nose Pliers 	- 12v air compressor	- Electrical circuit tester

- Square tipped screwdrive		- Torpedo level 		- Fuse Puller		- Tire Gauge

- Assortment of flat-head & Phillips  screwdrivers 		- Trailer jack		- Hammer & mallet

- Basic Wrenches, both box and open end 			- Booster cables		- Channel Locks

- Socket Set, if you're so inclined

(There is a TM Tool & Repair Kit suggestion in the TM Tips checklist that follows)

Nice to Haves

(  )  Propane stove (outside)

(  )  Folding Shovel

(  )  Axe

(  )  Sewer Hose Support

(  )  Regular Extension Cord - A 12-gauage, 20-amp, 25-foot.

(  )  Garden Hose Mending Fittings

(  )  Duct Tape

(  )  Electrician Tape

(  )  Bungee Cords

(  )  Another Flashlight

(  )  Trash Can

(  )  Coax Cable - For Cable TV.  You'll need this to avoid renting a cable from the campground, for those sites that have cable TV hookups.

(  )  Telephone and Telephone Wire - Some campgrounds have free local phone service, and it's nice to have a phone to call local attractions for more info, make reservations at a local eatery and these types of calls.  If you don't have a phone, you can always rent one from the campground.

(  )  Lubricating Oil - A can of WD-40 will do just fine.

(  )  Vacuum Cleaner

(  )  Broom and Dust Pan

(  )  Ant Spray and Ant Traps

(  )  Swiss Army Knife

(  )  Rope and/or String

(  )  Carpet or Fake Grass

(  )  Extra Mirror - just like at home, both of you are going to need the bathroom mirror at the same time.  Consider this an "argument-saver".

Initial Inventory

This checklist is a suggestion.  Customize it as you see fit.  It represents suggestions from numerous TM owners and other campers.

KITCHEN PANTRY

Cereal

Wine, Beer

Cookies

Garlic powder

Peanut butter

Pepper

Salt

Pam

Soup

Salad dressing

Spaghetti sauce

Jelly/jam

Sweetener

Canned biscuits

Tea bags

Juices

Coffee & coffee measure

Barbecue sauce

Crackers (snack, soda, graham)

Olive oil

Vinegar

KITCHEN PERISHIBLES

Bread

Ice

Tomatoes

Potatoes

Onions

Tuna

Milk

Meat

Butter

KITCHEN HARDWARE

Dish towels

Paper towels

Dish cloths

Aluminum foil

Wax paper

Saran wrap

Measuring spoons

Gas lighter

Paper products

Small baggies

Large baggies

Coffee pot

Toaster

Coffee Filters

Measuring cups

CLOTHING

Hats

Jackets/cool weather

Swimwear & shorts/warm weather

Ponchos/slickers for wet weather

Umbrellas

Activity clothes (hiking boots,etc

Socks

Underwear (bottoms & tops)

BATHROOM ITEMS

Linens

Bath towels

Wash cloths

Hand towels

Laundry

Large garbage/dirty clothes bags

Detergent

Clothes line, clips

CLEANING SUPPLIES

Cleanser (No bleach)

Glass cleaner

Broom & dust pan

Soap

Sponges

Rags

Detergent

Bleach

Softener

Scratcher pads

Clothes bag

Black tank chemicals

PERSONAL ITEMS

Toilet paper

Medicine

Neosporin ointment

Hydrocortisone ointment

Antacids, etc

Aspirin or other

Band Aids

Ace bandage wraps

4x4 gauze & tape

Sun block

Bug spray

Personal medications

Kleenex

Books, magazines

ELECTRONICS

Music & Movie CDs

Still camera

Battery charger

Batteries

BEDROOM

Bed linens

Sheets

Pillow cases

Pillows

Sleeping Bags

SETUP/ TAKEDOWN ITEMS

Copy of DL, insurance

Cards for camping discounts

Battery charged

3/8” drill, crank bit & chuck key

Hitch-pin locking ring and padlock

Water hose with nozzle

30-amp extension cords (2)

Leveling boards (2" x 8" x 5')

Stabilizer boards

TRAVELING ITEMS

Road atlas

Ice chest

Drinks / Water bottle

Snacks

Pillow for truck

Cell phone and power cord

SAFETY/ VEHICLE

Spare12 volt lamps/bulbs

Spare tire, truck -- proper inflation

Spare tire, trailer -- proper inflation

Toolbox

Spare fuses

4-way lug wrench

MISC

Extra keys

Outside rugs (2)

Outside door mat

Grill

LP Gas bottles for grill

Bucket

Fan(s

Lawn chairs

Games/decks of cards

Paper/pens/pencils/pencil sharpener

Picnic table cloth(s) and hold-downs

Fishing gear

Tire pump/Air compressor

Extra transmission fluid

Extra engine oil

Trailmanor Tips

Special thanks to Bob Strong for many of these suggestions.  The products listed below are those that Trailmanor owners have used, found to be effective, and have recommended them to others.  In most cases, less expensive alternatives are also presented for those of a “frugal” nature.

(  )  Cleaning the Outside Surfaces.  Scrubbing Bubbles works well.  Spray it on, let stand a few minutes, wipe off.  Does a good job on black streaks as well.  Other products to also consider: Purple Power (automotive stores), Kaboom (Dollar General Store), Greased Lightening (WalMArt), Soft Scrub (careful, because any kind of cleaner with bleach can take the color out of other material).

(  )  Waxing the Outside.  Protectal (Camping World or WalMart) works well.  Spray on, buff dry.  Excellent shine lasts through several washings, also prevents dirt buildup.

(  )  Recirculating Toilet.  This device has prompted more posts on the Trailmanor Owners Forum probably than any other subject.  The number one item for questions by new and prospective buyers.  Use only Thetford Campa Chem liquid.  Walmart sells it 6 bottles to a carton (better price than Camping World).  Also add Potty Toddy Tabs from Camping World -- 1 tablet per charge.  Using the combination of chemical and tablet should eliminate odors.  Keeping the number of times that “solids” are involved will also help (use the campground facility for that).  Also upgrade the pump fuse from 5 amp to 8 amp.  Some TM owners who are sensitive to the chemical smell that might be associated with the Campa Chem & Potty Toddy mix suggest using Odorlos from Camping World (say no smell or formaldehyde).

(  )  Cleaning the Curtains.  Use Tuff Stuff (avail at WalMart and automotive stores) upholstery cleaner.  Spray curtains while hanging.  Let dry & vacuum.  They’ll look new and no shrinking.

(  )  Beds.  Don’t waste time with sheets & blankets.  Buy a bed bag with sheets that Velcro inside (Coleman makes one big enough for two - Sierra Madre queen size bag for about $60).  If you buy sleeping bags, buy the kind that zip all the way open.  Zip them open, and store them on the bed like a comforter.  Saves time rolling them to store.  When you use them, zip into them like normal.

(  )  Inside Cleaning.  Greased Lightening works well on cleaning the top as well as the underside of the flaps.  Spray on, let soak, wipe off.  Spray the flaps now & then with Protectall, spray and bull (cleans and repels dirt.

(  )  Gray Water tank.  When hooked up at a site, it’s a good idea NOT to keep the gray water valve open.  By allowing the tank to fill and empty periodically, you can use the gray water to flush the sewer hose after black water dumps.  Additionally, by keeping the valve closed, you’ll eliminate sewer gases and smells from coming up through the drains into the TM.

(  )  Leveling.  There are two procedures for leveling.  The first uses blocks or levelers under the wheels and only use the stabilizer jacks to “stabilize” the floor load.  The second procedure for newer model TMs use the “scissors jacks” to actually level the trailer.  Regardless of method, be sure at least one wheel is chocked prior to disconnecting TM from TV and leveling.

1.  Using leveling blocks and stabilizing with the “stabilizer jacks.”  Pick up a pack of Lynx Levelers and a couple pressure-treated 1X4 about 2’ long.  Always level before opening up the TM.  Back into the site, place a torpedo level across the tongue of the TM on a flat spot behind the coupler hitch.  Level side to side first -- half bubble = one 1X4; whole bubble = one Lynx Leveler block.  Back the trailer up a little (about a foot), and place what ever was needed to level on the ground in front of the TM door side wheel.  Pull the TM back to where it was the first time (with wheel on block).  Check level again and it should be level.

Chock the other TM wheel, unhook from the TV, and place level on a frame rail by the propane tank, and check level fore to aft.  Use the tongue jack to level (be sure a pressure block is under tongue jack foot).  Lower all four corner stabilizers (pressure-treated block under foot), and crank down until just snug.  Check level again.  As an added level confidence, check level inside on a kitchen cabinet after setting up.

2.  Using “scissors jacks.”  Place block under tongue jack post, and level front-to-back using tongue jack.  After level is established front-to-back, check side-to-side level.  Place blocks under where all four scissors jacks will contact earth.  Crank down on the LOWEST corner until that side indicates level.  Check and readjust side-to-side level.  Crank down HIGH SIDE jacks until it bears weight.  Repeat for other two scissors jacks.  With all scissors jacks bearing weight, readjust side-to-side level.  Recheck front-to-back level, and adjust as necessary.  With all jacks supporting TM weight recheck level on both axis (front-to-back, and side-to-side).

Hint (1): anytime you adjust ANY jack, you need to recheck level on both axes

Hint (2): Level initially after you take the TM home.  Get it good and level and then install bubble levels at the corners to help in all future leveling.  Makes it much easier than using a torpedo level each time.

Hint (3): Why is leveling so important?  Besides the obvious… keeping you from rolling out of bed, or making cakes that are crooked, leveling is extremely important to the operation of the refrigerator.  The fridge is very prone to degraded operation if it is not level.

(  )  TM Tool & Repair Kit.  (Thanks to Bill & dennisu)

- Couple sizes of straight and Phillips screwdrivers, some pliers, wrenches, a small hammer.  Just make up what you would use at home.

- Bottle jack (hydraulic jack).  The TM does not come with a jack to change a flat on the TM, and your car's bumper jack won't work.  So buy a small bottle jack and toss it in the rear bumper compartment.

- Square-drive screwdrivers or square drive bits.  Once you have the TM jacked up, you can't get the wheel off until you remove the fender skirt.  And like many parts of the TM, the fender skirts are attached with square-drive screws.  Got a set of square-drive screwdrivers?  If not, get 'em.

- Socket set.  WalMart has a nice inexpensive socket set with both English and metric sockets, for less than $10.  It is in a green canvas zip-up case, and carries nicely.

- Spare fuses.  Make sure you have spare 12-volt fuses for every fuse you can identify.  Most are the automotive flat-blade fuses - get some rated at 10 amps, 15 amps, and 20 amps. The 12-volt main fuse is unique - a 30-amp glass tubular (cartridge) fuse.  Until you can replace that fuse holder with an automotive type, be sure you have a couple spare fuses for this location.  The 120V power side uses 15, 20, and 30 amp fuses.  Carry spares for these too.

- 12-volt test light is handy.  A multimeter or Digital Volt Meter (DVM) is even better.

- Spare bulbs for the interior and exterior lights.

- 10-foot length of #10 insulated wire, along with an assortment of crimp terminals, and a crimp tool.

- Wire cutter/stripper.

- Wire nuts, assorted sizes.

- Dozen nylon cable ties, various lengths.

- Two or three sizes of stainless-steel worm-type hose clamps.

- Duct tape.

- Box-cutter type of retractable knife.

- 12-volt air compressor and 20’ cord adapter.  Extension cord allows you to plug into the TV lighter plug and reach without having to unhook from the TV or open the TM to access the 12V lighter plug.

- Misc incidentals: Latex gloves.  Baby wipes.  A small can of 3-in-1 oil for the spring bars and a toothpaste-size tube of white grease to lube the hitch ball.  Cordless drill and 1/4" Allen wrench to crank the scissors jacks.

- Finally, always carry the TM Owner’s Manual, your checklists, and tool kit in the TV not the TM.  Makes access a whole lot easier when you are on the road.

(  )  Periodic Maintenance.  Besides checking tire condition, interior and exterior lights, brakes, appliances, battery, etc. for normal operation, the following periodic maintenance is suggested:

TIRES: 14” = 50 psi; 15” = 65 psi

PROPANE SYSTEM: Periodic leak check.

WHEEL LUGS: Tighten every 50 miles when new until 200 miles; then tighten every 500 miles thereafter.

WHEEL BEARING LUBE: Use lithium based wheel bearing lube; 10-20 pumps per zerec fitting every 2000 – 5000 miles; 30-40 pumps per fitting to completely recycle grease through wheel hub.

Water Heater Use Options

OPERATION USING PROPANE 
1. Hot water tank - FILL 
2. Turn on propane (gas) supply. 
3. Turn switch to ON; note amber light illuminates. 
4. Observe burner light off (amber light extinguishes) 
NOTE: If burner does not light on first try, the system will lockout. If lockout occurs before main burner lights, turn switch OFF for 5 seconds before turning switch to ON position. 
[Continue as written in manual]: The first start-up of the heater may require several ignition cycles before all air is purged from the gas lines. Etc.....

OPERATION USING ELECTRIC HEATING ELEMENT 
1. Hot water tank - FILL 
2. AC electrical power - ON 
3. Switch used for propane heating: Ensure - OFF 
4. Go OUTSIDE trailer to switch electric heating element ON 
5. Water heater access panel - REMOVE 
6. Cotter Pin-guarded ON / OFF switch - LOCATE 
7. Cotter pin - REMOVE (Use needle-nosed pliers) 
8. Switch: Press to-  ON 
NOTE: It is easy to misplace/drop the cotter pin when removed. 
Handle with care, and keep in a secure location.

OPERATION USING BOTH

You can operate using both propane and electric.  You will realize a faster recovery time on the hot water using both, just remember to turn them both off when you close up.

TURN WATER HEATER OFF 
1. Turn switch to- OFF 
2. Insert Cotter Pin (if electric element was in use) 
3. Propane supply- OFF 
4. Drain water heater if desired (necessary if freezing temps anticipated)

Trailmanor Improvement Projects

Hitch

- Install Reese or Draw-Tite weight distributing hitch.

Electrical

- Replace plastic wire nuts with water proof ones (break away switch junction, both wheels brakes.

- Make battery minder adapter to provide secure & safe connection via TM tow vehicle power cable.

- Add 2nd 12 volt battery.

- Add Link-10 battery monitor in 2” round opening to left of radio.

- Upgrade converter to well filtered 7300 series.

- Replace outdoor porch light with amber bulb.

- Clean up wiring using UV proof zip ties for a neater appearance.

- Add wiring mod to provide AC power when closed.

- Upgrade speakers to 2-way design.

- Replace ceiling bulbs with lower wattage.

- Install low drain florescent lights in place of 1 or 2 incandescent.

- Install 110 volt outlet under table.

- Create a chain light to hang over table and works off 12 volt.  The bulb is a 12-volt florescent equal to a 75 watt tungsten but using 25% power.

- Install battery powered “moon light” inside cabinets, wardrobe, & rear storage.

- Install outlet in hallway near bed (clock or elect blanket).

- Make 12-volt extension light that hangs under awning.  Plugs into 12-volt receptacle.

- Add Solar panels.

- Install battery isolator to isolate converter from tow vehicle.

Livability

- Add AM/FM/CD radio.

- Scotch guard furniture, curtains, carpeting.

- Install screen door.

- Add pictures on wall next to sink, bathroom wall by sink, above stove, side walls near front bed.  Attach with Velcro.  Make look like models in brochure.  Dresses up TM.

- Install indoor/outdoor thermometer next to radio.

- Install mirror on inside or outside of wardrobe door.

Bathroom

- Make a cover for vent on side of tub.  In fall, cold air comes through opening.  If it’s windy you can also feel air coming in.  Use ¼ bendable Styrofoam.  Mount with Velcro.  When you want to cover vent, just mount on Velcro.

- Mount Rubbermaid towel holders.  Base screws to the wall.  Towel holder slides over base.  You can fold bathroom walls without the bases interfering.

- Replace vinyl shower curtain with a nylon one that feels like cloth.  Dries well and does not get brittle.

- Trim bathroom cabinet doors so that they open fully.

- Add over the door towel hooks or bars.

- Trim shower curtain to length.

- Make hands-free shower head holder.  Use over the door hooks, dispenser, & shower holder.

Kitchen

- Glue Velcro to wall behind stove.  Glue Velcro to back of multi-hook utensil hanger from WalMart.  Hang pot holders, can opener, butane lighter, & other cooking utensils.

- Add paper towel holder next to sink.  Glue over-the-door hooks to back of towel holder & hang over top of bathroom wall.

- Add stove top cover.  Be sure there are rubber pieces on bottom to keep from bouncing around in transit.

- Build a simple hanging cabinet base to sit it in while in transit.  Use 2X2s for the sides.  Glue a 2X2 fairing strip on cabinet bottom.  Attach little rubber stops to the corners so that the cabinet stays in place.  Paint white.  Now there is room for an over counter plug-in light.  You can also install a paper towel holder all the way to the rear.

- Attach a spice rack on side of wardrobe wall.  Screw base hooks into wall and slide rack over bases.  Remove when traveling.

- Build & install drawers on roller slides in kitchen cabinets.

- Build microwave stand with drawer to fill space next to stove.

- Install level-type faucet.

- Glue plastic knife safe inside wardrobe or under sink.

Structure, Frame, and Body

- Caulk all seams underneath.

- Reinforce under large cabinet (across from bathroom door) to hold the weight of the microwave more securely.

- Glue bubble level to tongue right in front of jack mount.

- Install interior “rub strips” where ceiling halves touch.

- Add bike rack.

- Replace slippery round door knob with lever-type.

Plumbing

- Build “Hands-Free” Shower head holder.

- Add heater to gray water holding tank.

- Upgrade LP tanks to ones with visual gauge.

- Reroute LP hose from regulator to the trailer body connector.  This hose hangs down, & is lowest part of the trailer.

- Install hanger & route an LP gas line for an outdoor grill.  Also modify folding table to attach to this same hanger.

Appliances & Heater

- Add mud dauber screen kit to cover outside heater openings.

- If furnace cover constantly comes loose, drill holes larger and install nut, bolt, & lock washers.

- Install remote sensing thermometer in refrigerator.  RV Parts Outlet has a digital thermometer with remote sensor (Part No. 16-0170)

- Add solar fan to refrigerator compartment.   Fan avail from RV Parts Outlet by Valterra (Part No. 22-0157).

A/C and Ventilation

- Add Max-Air kits to roof vents.

- Upgrade A/C filter to an electrostatic one.

- Add an A/C shroud brace.

Safety

- Mount a small CO detector a few feet from smoke alarm.

- Add heatproof brace to center of stove grate so that it doesn’t sag under a heavy pot.

- Add step rug to entrance step to reduce slip hazard.

- Rounded wheel well modification (older models)

Winterizing Your TM

Note: The TM factory states that the TM requires no special tools or materials to prepare it for storage in freezing conditions.  Simple draining of the water system, as described in the Owner's Manual, is sufficient.  In fact, this is one of the main selling points of a TM.  However, several TM Owners who live and store their TMs in areas where the winter temperature goes well below freezing have suggested additional winterization steps “just to be sure.”   if you feel more comfortable knowing that you have blown out the water lines and/or added antifreeze to the water system, here are the steps.  In most cases these aren’t necessary, but owners have suggested them as a “belt and suspenders” approach to winterization when storing a TM outdoors in very cold (below zero) climate.

(  )  Drain and flush fresh water tank

(  )  Drain and flush gray water holding tank

(  )  Drain and flush black water holding tank

(  )  Drain and flush hot water tank

(  )  Open low-point drain lines under TM:

(  )  Open the hot and cold faucets (including external shower)

(  )  Uncap the low-point drain lines; let water drain completely

(  )  Recap Drain Lines

(  )  Close the hot and cold faucets

(  )  Blow out the water lines (optional)

(  )  Screw on compressed air adapter onto main fresh water inlet

(  )  Apply compressed air, keeping pressure to less than 20 psi

(  )  Open each faucet, one at a time, to force water out of lines

(  )  Make sure to open each faucet in the TM, including both inside and outside showers

(  )  Open hot water drain plug and blow out the water in this tank

(  )  Remove compressed air and compressed air adapter

(  )  Replace drain plug in hot water heater and close all faucets

(  )  Put Antifreeze in the water lines

Use RV Non-Toxic Antifreeze ONLY.  Approximately 1-2 gallons should be plenty (This is an alternate procedure to blowing out the water with compressed air.  If you don’t blow out the water, consider this as a precautionary alternative).

(  )  Make sure water pump is off and all faucets are closed

(  )  Close the Water Bypass Valve to your hot water tank.  NOTE: If you don't have a Water Bypass Valve for your hot water tank, seriously consider having one installed.  Otherwise, in addition to the water lines you will end up filling the hot water tank with antifreeze.  It's not harmful, but you'll use an additional six gallons of antifreeze.

(  )  Disconnect the water line where it joins the fresh water tank - or - If your TM is so equipped, shut off the valve coming from the fresh water tank and open the valve for the winterizing inlet line

(  )  Place the water inlet line (either from the main system or the winterizing line) into the jug or other source container with the RV antifreeze.

(  )  Switch on the water pump; it will start to suck up the antifreeze into the lines

(  )  Open each valve of each faucet slowly, one at a time, until the red antifreeze starts coming out.  Once flowing, shut off the faucet valve.
Note: If for some reason you can't drain the water lines using the low-point drain lines and you can't blow out the lines with compressed air, you will have to wait until the antifreeze pushes out the fresh water in the lines before closing the valve.  A rule of thumb; don't shut off the valve until the antifreeze starts coming out.

(  )  Remember to also do this for both inside and outside showers

(  )  Shut off the water pump

(  )  Remove the inlet line from the antifreeze, then either: Re-attach the water line to the fresh water tank - or - Close the winterizing inlet valve and reopen the fresh water valve, if your TM is so equipped.

(  )  Pour a small amount of antifreeze down each drain to ensure that the sink traps have antifreeze in them.

(  )  Pour a small amount of antifreeze in the toilet bowl also.  Note: Don't flush it into the holding tank.

(  )  Before closing up TM, spread softener towelettes around.

De-winterizing the Water System

Note: If you winterized the system with antifreeze, the pink antifreeze must be well-flushed before first use.

(  )   Fresh water tank: FILL to ¾ full

(  )  Fresh Water Tank: ADD 1 to 2 capfuls BLEACH

(  )  Shake unit well to mix bleach & water.

(  )  Trailmanor unit: OPEN

(  )  Water Heater Bypass: BYPASSED

(  )  Water Pump: ON

(  )  Water Taps: OPEN one at a time until water runs clear.  (Don’t forget showers inside & out)

(  )  Let sit overnight

(  )  City Water: CONNECT

(  )  Taps: FLUSH all for at least 5 mins each

(  )  Water Heater Bypass: FILL

(  )  Fresh Water Tank: DRAIN, & REFILL with fresh water, add 1 capful bleach

(  )  Taps: FLUSH second time

(  )  Water Sources: CLOSED

(  )  Drains: OPEN and drain system (open taps to break siphon)

Storing Your TM

The following items are useful when you will be storing your TM for any extended period of time, not just for the winter.

(  )  Drain and flush all holding, fresh water, and hot water tanks.

(  )  Drain all water lines, using compressed air or at least the low-point drain lines.

(  )  Close the LP tank valve(s).  If your LP tanks don't already have a cover consider purchasing at least a vinyl LP tank cover.

(  )  Disconnect shore electric power to TM.

(  )  Remove all batteries from the TM and store in a place where they won't freeze.  Batteries that have been frozen will never again hold a proper charge, and will likely have to be replaced.

(  )  Visually check your TM for anything that might need to be repaired.  Make sure you also check the roof.

(  )  Defrost and clean refrigerator.  Place open boxes of baking soda or crumpled newspapers in the compartments to eliminate odors during storage.

(  )  Remove all perishable items from the interior and anything susceptible to freezing, if storing during the winter months.

(  )  Use a desiccant and container inside to absorb moisture to prevent mold.

(  )  Cover all external outlets, such as furnace outlet, to prevent critter entry.

(  )  Thoroughly wash the interior and the exterior of your TM.  You might also consider shampooing the carpets inside and waxing the outside.

(  )  Close all windows; leave the roof vent(s) open ever so slightly for air circulation during storage.

(  )  Spread softener sheets throughout TM.

(  )  Cover the tires to prevent weather-checking.

(  )  Cover the roof air conditioner

(  )  Cover with a tarp to cover your TM during storage.  Make sure you invest in a good tarp that can breathe.  Do not use an non-breathing tarp.

New TM Acceptance Inspection

This checklist is a modification of a universal RV acceptance checklist available from Tom_boles@yahoo.com.  It was customized for Trailmanor use.

STARTING POINT

Useful tools -- flashlight with a strong beam, screwdriver with a straight, Philips and square bits, delivery invoice showing standard equipment and ordered options, and pictures from the glossy brochure will also help in your inspection.  You will need the understanding & cooperation of your dealer for this process, as it probably involves far more depth & detail than they normally anticipate and schedule.  Make sure they know about it in advance.  (You may get a better-prepared trailer if you do!)  At the very least, tell the person doing the walk through what you intend to do, give him or her a copy of this checklist and work with them to plan the time accordingly.  A sense of humor will also come in handy!

Try to avoid the “Ooh, Ahh, Oh, Boy!” reaction.  The whole intent of the acceptance is to ensure that you got a quality product for the money you paid.  The last place to find that something was assembled incorrectly , doesn’t work, or leaks is on that first camping trip, a million miles from the nearest TM dealer.  This inspection is intended to make you a discriminating consumer, who values the hard-earned money spent on a quality product.  Take your time, and INSPECT EVERYTHING.

Finally, to complete this inspection the following must be available: propane tanks full, shore power, city water and a hose, sewer connections, drain or a dump station, and fully charged battery must be connected as well.

The intent of this checklist is that at it’s completion you will ensure all systems function, the unit is free of manufacturing defects, and YOU can erect and close the TM, operate all its systems, level it, and hook it up to the tow vehicle by yourself.

OUTSIDE WALK AROUND

Plan on an hour for the walk around.  At this point, you're generally searching for anything that does not look right.

Roof Sealing & Fixtures - ladder and get on roof.

(  )  Seams, gaskets, and any holes: SEALED well

(  )  Shell shrouds & covers: INTACT, UNBROKEN, SEATED, & SEALED.  Proper polyurethane caulking should have been used to seal all places where the roof has been penetrated.

(  )  Air conditioners, vents, antennas, sewer vents, and side seams: SEALED

(  )  Roof surface: NO bubbles (large and small), dents,  delamination, foreign objects, protruding screw, or nail heads.

(  )  A/C Mount & vent areas:  No depression or “cave-ins”

(  )  Water test:  Spray water onto roof shells after opening and ensure no leaks around vents or A/C

(  )  Caulking:  Applied liberally, smoothly, with no gaps.  Ensure that caulking is not dripped and smeared excessively

Windows

(  )  ALIGNED, SEATED, & SEALED

(  )  Rock cover:  pins properly align

(  )  Screens:  Smooth & not torn

Entry Doors

(  )  Gasket and seals: proper adhesive, coverage, sealed, no tears

(  )  Door Halves: ALIGN

(  )  Alignment: sits FLUSH against INSIDE of JAMB

(  )  Locks: KEYS WORK

(  )  Operation: OPEN, CLOSE, & LOCK with out undue effort

(  )  Windows: OPERATE SMOOTHLY

Storage Compartments

(  )  Doors: ALIGNED, GASKETS aligned, open and close smoothly

(  )  Keys & Locks: WORK

(  )  Hinges: TIGHT & SECURE and latches hold the door tightly closed and still be easy to open

(  )  Floor and carpeting: DRY, free of any signs of moisture that might indicate rain leakage.

(  )  Compartment lighting (if fitted): WORKS

Sewer & Fresh Water Connections

(  )  Sewer & Fresh water area: nothing broken or deformed

(  )  Valves & fittings: UNDERSTAND OPERATION

(  )  Black and gray water valves: UNDERSTAND FUNCTION

(  )  Low point drains: LOCATE and UNDERSTAND operation

Telephone & Cable TV Connections- Find and understand the telephone and cable connections.

(  )  Connectors: WEATHER CAPPED & SEALED

Propane

(  )  Regulator: UNDERSTAND OPERATION & how it switches ylinders.

(  )  Pigtails & system: LEAK CHECKED

(  )  Main Shutoff Valve: LOCATE & OPERATE

Battery

(  )  Box: VENTILATED

(  )  Cables: NOT RUBBING any part of the frame because that will eventually end up with a short circuit and possible fire

(  )  Battery Type: UNDERSTAND type and how to maintain them

Paint & Siding

(  )  Finish: CHECK

(  )  Sides: CHECK for bumps, dents, depressions by siting down the side of the unit

(  )  Siding: INSPECT by dividing each side of the unit into 2, 3 or 4 sections and inspect for color variations, dents or irregularities

(  )  Ends: Same procedure as sides

(  )  Vinyl Graphics: Smooth & free of air bubbles

(  )  Decals: Edges NOT LIFTED

(  )  Painted graphics: FREE FROM OVERSPRAY where masked

(  )  Access panels:  Not warped and all hardware present

Tires and Wheels

(  )  Inflation: CORRECT PRESSURE

(  )  Lug Nuts: TORQUED

(  )  Jack: Verify type and jacking point locations

(  )  Lug Nut Wrench: VERIFY SIZE

(  )  Spare: CHECK condition & pressure

(  )  Spare Carrier: UNDERSTAND operation

(  )  Wheels:  Properly balanced

Open and Close Unit

(  )  Corner latches:  work properly, snap shut, all hardware present, mounted tightly

(  )  Hold Down latches:  Engage and disengage freely, all hardware tight

(  )  Corner Swing up latches:  Operate freely, pin aligns with hole in square bar

(  )  Roof shells:  Open and close without extensive effort.  (The effort needed to open and close roof shells is a function of the torsion bar adjustments.  If too much effort is required, adjust the torsion bars and try again)

(  )  Beds:  slide freely and latch properly

Awnings

(  )  Extension/Extraction: CORRECT OPERATION

(  )  Springs/Locks: FUNCTION PROPERLY and ALIGNED

(  )  Mounting Points: VERIFY SOLIDITY

Chassis Inspection - Put on some old clothes or coveralls and get a good-sized piece of cardboard or carpet to make it easier to lie on you back while checking around under the trailer. If it's possible to do so without jacking up the rig, it's a lot easier, but do what makes sense to you.

(  )  Wiring: SECURED, property tied, & not rubbing

(  )  Frame:  Straight and not sagging.

(  )  Finish:  Rust free and evenly applied.

(  )  Hardware:  All mounting hardware tight

INSIDE FIT AND FINISH

Now its time to go inside. In general you are looking for things that are not finished correctly since it is really too late to inspect the design of anything.

Cabinets -

(  )  Inspect & open all of the cabinets to insure that all the hinges and latches work well.

(  )  Drawers: Pull each drawer out to its stop, return it closed and then try to open it like road vibration might do.

(  )  Pipes/plumbing/wiring: No water leaks and that all the wiring and pipes are well fastened.

(  )  Linings: FASTENED SECURELY

(  )  Finish: CHECK for delamination or loose edges on molding or vinyl wraps (you may want to wear gloves for this part)

(  )  Wardrobe: Open and close all closet doors checking for free operation and proper alignment.  Hanger rods should be properly fastened and secure.

Molding & trim - Go over all the trim on walls, doors & furniture.

(  )  Fit: Fastened on well and not loose or ready to come off

(  )  Nails/staples: None protruding

(  )  Velcro:  Properly applied throughout

(  )  Gap seals:  All present and properly mount to velcro

Lighting - Operate every light switch and observe its function.

(  )  Monitor Panel: CHECK battery level

Battery Disconnect Switch: LOCATE and verify operation

NOTE: At this point, only 12V lighting can be tested, as the unit is NOT connected to shore power yet

Furniture - Examine every piece of furniture to check for construction, upholstery, pattern and cloth matching.

(  )  Dinette: MAKE INTO BED

(  )  Sofa(s): MAKE INTO BED

(  )  Dinette table:  Swings and mounts properly

Drapes - Check for alignment.

(  )  Valences: SECURE

Counter Tops - Inspect all counter tops for alignment and fastening.

(  )  Trim: TIGHT

(  )  Caulking: PROPERLY APPLIED

(  )  Sink & Faucets: PROPERLY INSTALLED

(  )  Counter tops:  No delamination or gluing voids

Windows - Open and close every window and operate the latches.

Pay particular attention to the safety egress windows or emergency windows and make sure they operate smoothly & easily.

(  )  Hardware:  All screws and hardware properly installed, no missing knobs

Floor Coverings - Inspect carpet and other floor coverings in all corners to insure that they have been properly fastened down.

(  )  Tile: FREE FROM GOUGES/

(  )  Carpeting:  Properly installed, stain-free

Wall Coverings - Check to make sure that all the wall coverings actually cover and join properly.  Look for any discoloration or patch jobs that may cover hidden problems.  Try to find any places where it is not perfect since now is the time to get it fixed while matching patterns are in stock.

(Some folks suggest ordering extra fabric, carpet, and wallpaper now so that matching material is available to make small modifications or repairs at a later date.)

Sinks, shower tub, toilet:

(  )  Finish:  No scratches, gouges, or cracks

OPERATION TEST OF ALL HOUSE SYSTEMS

You should be about two hours into to the PDI by now and you are ready to test all of the house type systems.

Shore Power System- Now is the time to connect up to shore power.

(  )  AC cord: Pulls out correctly, , confirm the length and inspect the plug for proper attachment

(  )  Inside the unit: CHECK any 110V lighting and switches.  (If you have a 110V receptacle tester, check all outlets for proper wiring/polarity.)

(  )  GFI-protected outlets: TEST,  Understand which receptacles "down stream" from the GFI are protected.

Any electrical problems here should be corrected immediately.

Converter - Confirm the operation of the converter/charger that is installed.

(  )  Turn on several interior lights to create a load for the converter and confirm no excessive converter noise or vibration.

(  )  Have the PDI person explain the operation of the converter, the AC circuit breakers and the DC fuses.

(  )  Make sure that there is a written list of the loads connected to each fuse or breaker.

Water Pump - Fresh water tank needs to be full so you can test the function of the water pump.

(  )  After turning it on, you should hear it pump for several seconds, even up to a minute to create enough pressure in the system.

(  )  If the pump does not shut off, there is a problem.

(  )  Run water in the kitchen and bathroom sink and notice that the pump will come back on until proper water pressure is restored.

(  )  Now is the time to fix a noisy pump if it is vibrating or making any irritating sounds.

City Water System - Turn off the pump, connect up to city water (use a regulator if overly high pressure is suspected) and confirm that the connection works correctly.

(  )  Leaks: NONE

(  )  Fixtures: CONFIRM OPERATION

Gray Water Tank - Re-fill the fresh water tank if necessary and run water into the gray tank to verify the gauge reading and that there are no leaks.

(  )  Tank: FILL until water comes up in the shower will make it easier to find leaks inside & out.

(  )  Gauges: CHECK accuracy

(  )  Leaks: FIX

(  )  Tank: DRAIN, after toilet is checked, using sewer hose

(  )  Drains:  Tight and no leaks

Toilet

(  )  Toilet: CHARGE, ensure pump primes, flush, ensure water circulates

(  )  Leaks: NONE

(  )  Tank: RINSE & Drain

Water Heater - Try the water heater on propane first.  Ensure propane tanks are on.

(  )  A few seconds after you turn it on, you should hear the click of the igniter and the small pop when the burner lights.

(  )  The red light should stay on until that process happens.

(  )  If it does not ignite, then there is a problem.

(  )  AC: Switch to AC and confirm operation

(  )  Make sure you know the location of the electrical switch at the water heater and the correct operation of the interior switch.

(  )  Bypass valves: UNDERSTAND operation for winterizing

(  )  Confirm that hot (or warm water depending on how long the water heater has been on) comes out of the hot tap at the various sinks.

Furnace - Now its time to understand the operation of the thermostat that controls heating and sometimes the air conditioning.

(  )  Furnace: ON and set a temperature demand that is at least 10 degrees hotter than ambient temperature.  In about 30 seconds, you should hear the furnace fans come on.

(  )  Shortly thereafter you should hear the click of the igniter and the sound of the burner. If not then there is a problem. It could also be taking a while for propane to get to the heater, so don't despair.

(  )  Let the furnace blow and you should get hot air at about 110 degrees coming out of all vents. Check each one.

(  )  Now turn the furnace down and the hot air will gradually turn cooler and the fans will eventually stop after the furnace has cooled sufficiently.

(  )  During this process have someone with a good nose checking for any smell of material getting too hot, or exhaust coming out.

Air Conditioner - Some air conditioners also have a heat strip or heat pump feature so now is the time to verify these functions.

(  )  A/C: ON

(  )  After a couple of minutes, cool air, 20 degrees cooler than ambient, should be coming out vent

(  )  Learn how to clean the filters at this time.

(  )  Leaks:  None, either from condensation or around mount seals

Propane and Carbon Monoxide Alarms  & Smoke Detector - The PDI person should have a small canister of gas that can be sprayed at the alarm to test its operation.  Have them perform this test while you watch and learn how the alarms work.

(  )  Smoke Alarm: NEW BATTERY, TEST

Refrigerator - Most modern refrigerators work on propane or AC and have an automatic mode that gives preference to AC and then will switch to propane if AC power is not available.

(  )  Understand the controls and the status lights and set the unit on propane. The TM may need to be unplugged for this to happen.

(  )  Go outside to the refrigerator vent grill and make sure that the propane heating column is lit and heating.

(  )  While the refrigerator grill is open, check to make sure the drain line is positioned for proper drainage.

(  )  Set the temperature at the highest cooling setting, because setting it to lowest will typically cause the coils to collect moisture and ice up.

(  )  Come back in about 10 minutes to feel that the coil/fins is actually starting to cool.

TV Antenna and Switching - For the most through test, bring a small AC/DC TV with you for the initial test

(  )  Review and understand the switching system that allows the selection of viewing channel on the front and rear TV

(  )  Raise the TV UHF/VHF antenna and learn how to turn on the amplifier and the DC outlet.

(  )  Find out the power rating of the DC outlet and compare it to the load of the equipment you will connect to it.

(  )  Activate the control on the front TV that scans for local stations.

(  )  Learn how to rotate the antenna to maximize the quality of the picture.

Air Vents - Test the operation of kitchen and bathroom air vents making sure they open and turn on properly (if powered). Verify that they retract and close tightly.  Check any other vents for proper operation.  Ensure the seals make them leak free.

Propane Stove - Turn on one burner of the stovetop while the AC's and Microwave are running and the automatic igniters should cause a strong spark to light the burner.

(  )  Turn on the other burners to verify that there is enough propane flow to operate the refrigerator, water heater and all the burners.

(  )  If everything is OK, turn off the burners on the stove.

(  )  If your unit has an oven, then understand how the pilot is lit and verify its operation now.

Note: Sometimes the burner igniters interfere with the operation of the thermostats for the roof AC. This is the time to find this problem.

Summary

You have now done a simple test of the major house systems and can shut everything off.  By now, you will have a list of things that you feel need correcting, but it may be then the end of the day.  Decide if you are going to accept the trailer as is or if it needs to stay with the dealer. If it is appropriate, plan on camping near the dealer for the night. This will give you a chance to further test the various functions.  Prepare yourself for waiting until these things are corrected and don't be tempted by the PDI person to sign the acceptance paperwork just yet. For a really serious checkout of your new trailer, you should "dry camp" the first night and not be tempted to hook up to shore utilities just yet. Bring enough kitchen equipment and food so that you can prepare an evening meal. There is no better way to test the living facilities than to actually use them. Don't be tempted, since you've had a tough day to go out to eat.

HITCH & BRAKE CONTROLLER

If you bought W/D hitch & brake controller from the same dealer, be sure they explain and demonstrate operation, hookup, and adjustment.  Be sure you know how to hook up and adjust the W/D hitch properly.  Be sure that the brake controller is adjusted to the manufacturers specs.  Drive the unit hooked up, and operate the brake controller.

ACCEPTANCE

When you are satisfied that all systems are "go" then sign the acceptance papers that the PDI person will anxiously provide for you. Schedule your first return trip to the dealer for about a week to a month from now. You are now ready to take your "shake down" cruise.

SHAKE DOWN CRUISE

Select an interesting destination about 100 miles away for your maiden voyage. Actually use all the systems multiple times to try to detect and infant failures (electronic equipment fails at greater rates at the beginning of its life). Carefully note any problems or discrepancies in a list and make a copy for the dealer when you take the trailer in again.

### Equipment List

Ed Felker’s “Snowball List.”  Offered as a suggestion to help others decide what to add to improve their TM.

| TrailManor Optional Equipment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Low Profile Air Conditioning | Screen door |  | A/C heat strip |  |  |
| TV Antenna w/amp | 7300 Converter |  | Awning |  |  |
| Hanging cabinet | Privacy curtain (wardrobe area) |  |  |  |  |
| ITEM |  |  | SOURCE |  | COST |
| Tow Equipment (W/D Hitch) |  |  |  |  |  |
| Draw-Tite w/10” shank (10,000lb/1000lb tongue) |  |  | E-3203-10 |  | 275.95 |
| or |  |  |  |  |  |
| Eaz-Lift 10000lb/750 lb tongue wt |  |  | RVO-05-0004 |  | 239.95 |
| 2” Ball |  |  | E-19263 |  | 14.95 |
| Ball Mount |  |  |  |  |  |
| Tow Equipment (Mirrors) |  |  |  |  |  |
| McKecsh Mirrors |  |  | H |  | 118.90 |
| Brake Controller Components |  |  |  |  |  |
| Prodigy Brake Control |  |  | R |  | 98.99 |
| Controller 7-Wat Installation Kit |  |  | E-39507 |  | 49.95 |
| Near Term |  |  |  |  |  |
| Sleeping Bag (Sierra Madre, double, mild) |  |  | C (2 each) |  | 122.00 |
| Potty Toddy Tablets |  |  | RVO |  | 20.69 |
| Electrical Adapter (30Af/15Am) |  |  | RVO-05-0031 |  | 2.69 |
| 25’ 30 A extension cord |  |  | RVO-13-0254 |  | 22.95 |
| 10’ 30A extension cord |  |  | RVO-03-0402 |  | 19.54 |
| CAMCO Hose Saver Elbow |  |  | RVO-03-0069 |  | 4.04 |
| RKRP EZ Ramp Levelers |  |  | RVO-20-0311 |  | 29.74 |
| CAMCO Wheel Chock |  |  | RVO-03-0111 |  | 4.49 |
| Electrical Adapter (15Am/30Af) |  |  | RVO-05-0032 |  | 2.69 |
| Water pressure regulator |  |  | RVO-13-0016 |  | 8.09 |
| ADCO Cover for folding trailer (12252) |  |  | CW-24423 |  | 188.10 |
| Other Equipment (long term) |  |  |  |  |  |
| Hitch Cover (PSU) |  |  | E-034092100133 |  | 14.95 |
| Hitch Cover (AF Retired) |  |  | E-034092101420 |  | 29.95 |
| Sink Set Ultra |  |  | CW |  | 5.39 |
| Turf Site rug |  |  | CW |  | 26.98 |
| Prest-O-Fit Step Rug |  |  | RVO-16-0022 |  | 10.20 |
| CAMCO Lantern Snaps (2 sets) |  |  | RVO-03-0093 |  | 6.50 |
| Valterra Solar Frig Fan |  |  | RVO-22-0157 |  | 44.99 |
| CAMCO Frig safety rails |  |  | RVO-03-0026 |  | 5.39 |
| Indoor Outdoor digital thermo/clock |  |  | RVO-16-0200 |  | 15.10 |
| Coach Lites |  |  | CW |  | 8.99 |
| Screen House (14X12) |  |  | W |  | 49.43 |
| Hydrolife Filter Kit |  |  | RVO-08-0951 |  | 54.95 |
| BAL Light Trailer tire leveler |  |  | CW-21017 |  | 58.49 |
| CAMCO Refrig ALarm |  |  | CW - 21870 |  | 17.99 |
| CAMCO Sink Cover |  |  | RVO-23-0026 |  | 28.04 |
| Hamper Bag |  |  | RVO-16-0225 |  | 7.50 |
| Stanley Water Thief |  |  | RVO-19-0001 |  | 2.69 |
| US Catalytic 4100 Tabletop BBQ |  |  | RVO-21-0011 |  | 76.45 |
| Single handle kitchen faucet |  |  | RVO-16-0007 |  | 48.44 |
| Valterra Water Heater Bypass |  |  | RVO-22-0171 |  | 17.99 |
| Blow Out Plug |  |  | RVO-CO21363 |  | 2.50 |
| Winterizing Pump Conversion Kit |  |  | RVO-22-0050 |  | 13.04 |
| SHURFLO Accumulator Tank |  |  | RVO-19-0155 |  | 32.29 |
| Solar Equipment |  |  |  |  |  |
| 75W, 4.4 Amp, 12 lbs (Kit) |  |  | RVSE - SPPKIT |  | 495.00 |
| C - Coleman |  | E - etrailer |  | RVO - RV Parts Outlet |  |
| CW - Camping World |  | R - RVWholesalers |  | W - WalMArt |  |
| H - Hensley |  | RVSE - RV Solar Electric |  |  |  |